Department of Electrical and Electronics Engineering

SRM IST, Kattankulathur – 603 203

Sub Code & Name: 18EEC301J – Control Systems

| Experiment No | 03 |
| --- | --- |
| Title of Experiment | Generation Of Standard Test Signals |
| Name of the candidate | ADARSH GOENKA |
| Register Number | RA1811005010037 |
| Date of Experiment | 26.08.2020 |

Mark Split Up

| S.No | Description | Maximum Mark | Mark Obtained |
| --- | --- | --- | --- |
| 1 | Oral Viva / Online Quiz | 5 |  |
| 2 | Execution | 10 |  |
| 3 | Model Calculation / Result Analysis | 5 |  |
| Total |  | 20 |  |

Staff Signature with date

AIM:

To generate the following test signals

- Step

- Ramp

- Parabolic

- Impulse

- Sinusoidal    with MATLAB-SIMULINK software.

APPARATUS/SOFTWARE/EQUIPMENTS REQUIRED:

- Computer/Desktop with MATLAB ( Standard Versions) Package

THEORY:

REF:http://engineering.electrical-equipment.org/panel-building/typical-test-signals-in-time-domain-analysis.html

- The dynamic behavior of a system is manipulated and judged under the application and working of standard and typical test signals.

- The characteristics of an input signal are constant acceleration, constant velocity, a sudden change or a sudden shock.

- There are four types of typical test signals: Impulse Step, Ramp, Parabolic and another important signal is sinusoidal signal

- STEP SIGNAL

- The step signal defines the sudden change in properties of actual signal.

- When A=1, the step is called unit step signal.

- RAMP SIGNAL

- The ramp signal tells you the constant velocity attribute of actual input signal.

- When A=1, ramp signal is called unit ramp signal.

- PARABOLIC SIGNAL

- Parabolic signal gives the constant acceleration distinction of actual input signal.

- When A=1, the parabolic signal is called unit parabolic signal

- IMPULSE SIGNAL

- Impulse response in control system imitates sudden shock quality of actual input signal.

- When A=1 then the impulse signal is called Unit impulse signal.

PROCEDURE:

- SIMULINK MODELING

- Click open the shortcut to MATLAB icon on desktop of computer.

- On the top tool bar, click the Simulink library browser icon.

- Open the new model from the library browser window’s top tool bar.

- Drag and drop the following blocks from the library to the new file.

- Simulink--- Sources---Step

- Simulink---Sources-Ramp

- Simulink-Sources-Sinusoidal

- Simulink---Sinks---Scope

- Simscape---Powerlib- Powergui

- Connect the blocks as per schematic connection diagrams given below.

- STEP, RAMP AND SINUSOIDAL TEST SIGNAL GENERATION

SAVE THE FILE BY GIVING A NAME

RUN

YOU COULD CHANGE VALUES

Click on curve to change colour and thickness of line

CHANGE TO WHITE

EDIT AXES PROPERTIES

PRINT TO FIGURE

Model Outputs:

- m-FILE CODING

- PARABOLIC AND IMPULSE TEST SIGNAL GENERATION

- Click open the shortcut to MATLAB icon on desktop of computer.

- On the top tool bar, click the new icon to open editor window as shown

- Key in the program.

- Save the file and run the program

EDITOR WINDOW FOR CODING

WORKSPACE

COMMAND WINDOW

UNIT STEP

UNIT IMPULSE

UNIT RAMP

UNIT PARABOLIC

Program/m-code

Simulation Output Waveforms:

Using Simulink:

Using M file coding:

RESULT:

Thus the various standard test signals are generated, observed and plotted by programming and modeling with MATLAB.

VIVA –VOCE

- Define time response.                                                                                         Time response of the system is defined as the output of a system when subjected to an input which is a function of time.

- Distinguish between transient and steady state response.

The steady state is the state that is established after a certain time in your system. 
The transient state is basically the state between the beginning of the event and the steady state.

- List the standard test signals.

Standard input signals are:

1) Impulse

2) Step

3) Ramp

4) Parabolic input

- What do you mean by a step signal?

A signal with magnitude one for time greater than zero .

- Mathematically formulate a unit step signal.

- What is ramp signal?

A signal whose magnitude increases same as time . It can be   
                 obtained by integrating unit step.

- Draw the graphical representation of a ramp signal.

- Define parabolic and impulse signals.

Impulse : A signal which has infinite magnitude at time equal to zero only. We can assume it as a lightning pulse which acts for a short duration with infinite magnitude of voltage.

Parabolic :A signal whose magnitude increases with the square of time . It can be obtained by integrating unit ramp

- Express the mathematical model of an impulse signal.

- Match the following

- Sudden change –Impulse

- Constant velocity- Step

- Sudden shock- Parabolic

- Constant acceleration- Ramp

- Sudden change –step

- Constant velocity- ramp

- Sudden shock- impulse

- Constant acceleration- parabolic